CasparCG Production Server

Complete Setup Guide — Real-World Verified

Ubuntu 24.04 LTS  •  NVIDIA RTX  •  DeckLink 8K Pro  •  NDI SDK v6  •  Media Scanner

ℹ NOTE: This guide is based on a real production setup session. Every command has been tested and verified working. Follow steps in order.

⚠ WARNING: CasparCG MUST run inside an X11 session started by LightDM/Openbox as the casparcg user. Never start it from SSH or TTY. Never run as root. Never use a systemd service for CasparCG.

Boot Sequence

Boot → NVIDIA → LightDM → casparcg autologin → Xorg :0 → Openbox → CasparCG + Scanner

# 1. BIOS Configuration

Before installing Ubuntu, configure BIOS for broadcast stability:

- Disable C-States

- Disable CPU power saving / ASPM

- Disable Secure Boot

- Set Primary GPU = PCIe

- Disable onboard GPU

- Set power profile = Performance

# 2. Ubuntu 24.04 Base Installation

## 2.1 Install Ubuntu Server

Install Ubuntu 24.04 LTS. During installation:

- Choose Ubuntu Server (not Desktop)

- Create admin user named 'serwer' — NOT 'casparcg'

- Enable OpenSSH during install

- Do NOT install any desktop environment

## 2.2 Initial Update and Tools

👤 Run as: root/serwer

sudo apt update && sudo apt upgrade -y sudo apt install -y curl wget git nano htop net-tools

# 3. NVIDIA Driver

## 3.1 Install Driver

👤 Run as: root/serwer

sudo apt install -y ubuntu-drivers-common ubuntu-drivers devices # Install the recommended driver shown above (e.g. nvidia-driver-590) sudo apt install -y nvidia-driver-590 sudo reboot

## 3.2 Verify and Enable Persistence

👤 Run as: root/serwer

# Verify driver loaded correctly nvidia-smi # Enable persistence mode sudo nvidia-smi -pm 1 # Enable persistenced service sudo systemctl unmask nvidia-persistenced sudo systemctl enable nvidia-persistenced sudo systemctl start nvidia-persistenced systemctl status nvidia-persistenced # Must show: active (running)

## 3.3 Make Persistence Survive Reboot

👤 Run as: root/serwer

sudo nano /etc/rc.local # Paste this content: #!/bin/bash nvidia-smi -pm 1 exit 0 # Save, then make executable: sudo chmod +x /etc/rc.local sudo systemctl enable rc-local sudo systemctl start rc-local

# 4. DeckLink Driver Installation

## 4.1 Download and Install

👤 Run as: root/serwer

cd /tmp wget -O decklink.tar.gz \ 'https://swr.cloud.blackmagicdesign.com/DesktopVideo/v15.3.1/Blackmagic_Desktop_Video_Linux_15.3.1.tar.gz' tar -xzf decklink.tar.gz ls Blackmagic_Desktop_Video_Linux_15.3.1/ # Install the .deb package sudo dpkg -i Blackmagic_Desktop_Video_Linux_15.3.1/deb/x86_64/desktopvideo_*.deb # Install gui version also for initial config sudo dpkg -i Blackmagic_Desktop_Video_Linux_15.3.1/deb/x86_64/desktopvideo-gui_*.deb sudo apt install -f # Load kernel module sudo modprobe blackmagic_io

## 4.2 Verify DeckLink

👤 Run as: root/serwer

lspci ¦ grep -i blackmagic # Expected: Blackmagic Design DeckLink 8K Pro lsmod ¦ grep blackmagic # Expected: blackmagic_io ls /dev/blackmagic/ # Expected: io0 io1 io2 io3

ℹ NOTE: The DeckLink devices appear as /dev/blackmagic/io0 through io3 for the 8K Pro (4 ports). These are readable/writable by all users (crw-rw-rw-).

# 5. NDI SDK v6 Installation

## 5.1 Download and Install

👤 Run as: root/serwer

cd /tmp wget -O ndi-sdk.tar.gz \ ‘https://downloads.ndi.tv/SDK/NDI_SDK_Linux/Install_NDI_SDK_v6_Linux.tar.gz' # (use the full URL from get.ndi.video — it redirects to downloads.ndi.tv) tar -xzf ndi-sdk.tar.gz # Extracts: Install_NDI_SDK_v6_Linux.sh chmod +x Install_NDI_SDK_v6_Linux.sh sudo ./Install_NDI_SDK_v6_Linux.sh # Accept the license agreement when prompted

## 5.2 Install NDI Libraries to System

👤 Run as: root/serwer

# Copy x86_64 library to system path sudo cp '/tmp/NDI SDK for Linux/lib/x86_64-linux-gnu/libndi.so.6.3.1' \ /usr/lib/x86_64-linux-gnu/ # Create symlinks sudo ln -s /usr/lib/x86_64-linux-gnu/libndi.so.6.3.1 /usr/lib/x86_64-linux-gnu/libndi.so.6 sudo ln -s /usr/lib/x86_64-linux-gnu/libndi.so.6.3.1 \ /usr/lib/x86_64-linux-gnu/libndi.so sudo ldconfig # Verify ldconfig -p ¦ grep libndi # Expected: # libndi.so.6 => /lib/x86_64-linux-gnu/libndi.so.6 # libndi.so => /lib/x86_64-linux-gnu/libndi.so

## 5.3 Copy NDI Library to CasparCG Working Directory

👤 Run as: root/serwer

# CasparCG loads libndi.so.6 from its working directory /opt/casparcg/ # This step is done after CasparCG is installed (Section 6) sudo cp /usr/lib/x86_64-linux-gnu/libndi.so.6 /opt/casparcg/ sudo chown casparcg:casparcg /opt/casparcg/libndi.so.6

# 6. Desktop Environment (LightDM + Openbox)

## 6.1 Install LightDM and Openbox

👤 Run as: root/serwer

# Disable GNOME if present sudo systemctl disable gdm 2>/dev/null ¦¦ true sudo apt install -y lightdm openbox # Set LightDM as default display manager sudo dpkg-reconfigure lightdm # Select: lightdm

## 6.2 Disable Display Sleep System-Wide

👤 Run as: root/serwer

# Disable console blanking at kernel level sudo nano /etc/default/grub # Find GRUB_CMDLINE_LINUX_DEFAULT and add consoleblank=0: # GRUB_CMDLINE_LINUX_DEFAULT="quiet splash consoleblank=0" sudo update-grub # Disable sleep targets sudo systemctl mask sleep.target suspend.target hibernate.target hybrid-sleep.target

## 6.3 Configure LightDM Autologin

👤 Run as: root/serwer

sudo nano /etc/lightdm/lightdm.conf # Paste: [Seat:*] autologin-user=casparcg autologin-user-timeout=0 user-session=openbox xserver-command=X -s 0 -dpms

ℹ NOTE: xserver-command=X -s 0 -dpms disables display sleep at the X server level — essential for broadcast use.

# 7. CasparCG Server Installation

## 7.1 Create casparcg User

👤 Run as: root/serwer

sudo adduser casparcg # Set a password when prompted sudo usermod -aG video casparcg sudo usermod -aG audio casparcg sudo usermod -aG render casparcg

⚠ WARNING: Do not manually create /home/casparcg. The 'adduser' command creates it correctly with proper permissions.

## 7.2 Install Dependencies

👤 Run as: root/serwer

sudo apt install -y \ ffmpeg \ fonts-liberation \ libavdevice60 \ libavfilter9 \ libavformat60 \ libboost-context1.83.0 \ libboost-coroutine1.83.0 \ libboost-filesystem1.83.0 \ libboost-locale1.83.0 \ libboost-log1.83.0 \ libboost-thread1.83.0 \ libglew2.2 \ libopenal1 \ libopengl0 \ libsfml-graphics2.6 \ libsfml-system2.6 \ libsfml-window2.6 \ libswscale7 \ libtbb12

## 7.3 Install CasparCG Server and Scanner

👤 Run as: root/serwer

cd /tmp # Download CasparCG server .deb # (obtain from https://github.com/CasparCG/server/releases) sudo dpkg -i casparcg-server-2.5_2.5.0.stable-noble1_amd64.deb # Download scanner .deb wget -O scanner.deb \ 'https://github.com/CasparCG/media-scanner/releases/download/v1.3.4/casparcg-scanner_1.3.4-ubuntu1_amd64.deb' sudo dpkg -i scanner.deb # Fix any remaining dependency issues sudo dpkg --configure -a sudo apt install -f

ℹ NOTE: After install: server binary = /usr/bin/casparcg-server-2.5 ¦ scanner binary = /usr/bin/casparcg-scanner ¦ working directory = /opt/casparcg/

## 7.4 Verify Working Directory Structure

👤 Run as: root/serwer

ls -la /opt/casparcg/ # Expected directories: cef-cache data log media template # Fix ownership if needed sudo chown -R casparcg:casparcg /opt/casparcg/

## 7.5 Remove Any systemd CasparCG Service

👤 Run as: root/serwer

# CRITICAL: Check for and remove any systemd service sudo systemctl stop casparcg-server 2>/dev/null ¦¦ true sudo systemctl disable casparcg-server 2>/dev/null ¦¦ true sudo rm -f /etc/systemd/system/casparcg-server.service sudo systemctl daemon-reload # Verify no casparcg service exists systemctl list-units ¦ grep -i casparcg # Should only show: session-c1.scope and syncthing@casparcg (if installed)

⚠ WARNING: A systemd service running CasparCG alongside the Openbox autostart causes a 'port 5250 already in use' crash on every boot and after every RESTART command.

# 8. CasparCG Configuration

## 8.1 Create Working Config

Switch to the casparcg user to create config files:

👤 Run as: root/serwer

sudo -u casparcg bash # You are now running as the casparcg user

👤 Run as: casparcg

nano /opt/casparcg/media/casparcg.config.ftd

Paste this configuration (your client app will overwrite it via DATA STORE):

<?xml version="1.0" encoding="utf-8"?> <configuration> <paths> <media-path>media/</media-path> <log-path disable="false">log/</log-path> <data-path>media/</data-path> <template-path>template/</template-path> </paths> <lock-clear-phrase>secret</lock-clear-phrase> <channels> <channel> <video-mode>1920x1080</video-mode> <consumers> <screen> <device>1</device> <stretch>fill</stretch> <windowed>true</windowed> <vsync>true</vsync> </screen> </consumers> </channel> </channels> <controllers> <tcp> <port>5250</port> <protocol>AMCP</protocol> </tcp> </controllers> <amcp> <media-server> <host>localhost</host> <port>8000</port> </media-server> </amcp> <ndi> <name>CasparCG-Ch1</name> <allow-fields>false</allow-fields> </ndi> </configuration>

ℹ NOTE: The <amcp><media-server><port>8000</port> tells CasparCG where the media scanner API runs. The scanner MUST listen on port 8000. This is separate from the AMCP port 5250.

# 9. Media Scanner Configuration

The scanner reads its config from ./casparcg.config in the current working directory. It expects XML format. It must listen on port 8000 (not 5250).

👤 Run as: casparcg

# Make sure you are the casparcg user: # sudo -u casparcg bash nano /opt/casparcg/media/casparcg.config

<configuration> <paths> <media-path>/opt/casparcg/media/</media-path> <log-path>/opt/casparcg/log/</log-path> <template-path>/opt/casparcg/template/</template-path> </paths> <scanner> <host>localhost</host> <port>8000</port> </scanner> </configuration>

⚠ WARNING: The scanner ignores any config path argument — it ALWAYS reads from ./casparcg.config in the current working directory. Always launch it with 'cd /opt/casparcg/media/media/media' first.

⚠ WARNING: Scanner port MUST be 8000. CasparCG server uses port 5250 for AMCP. These are different services on different ports. Never set both to the same port.

# 10. Openbox Autostart

## 10.1 Create Openbox Config Directory

👤 Run as: root/serwer

sudo -u casparcg bash # Now as casparcg user:

👤 Run as: casparcg

mkdir -p /home/casparcg/.config/openbox

## 10.2 Optional: Center Screen Consumer Window

To automatically center the CasparCG windowed output (useful during development):

👤 Run as: casparcg

cp /etc/xdg/openbox/rc.xml /home/casparcg/.config/openbox/rc.xml

## 10.3 Create Autostart Script

👤 Run as: casparcg

nano /home/casparcg/.config/openbox/autostart

Paste the complete autostart:

#!/bin/bash exec >> /tmp/autostart-debug.log 2>&1 echo "$(date): Autostart beginning" export DISPLAY=:0 export XAUTHORITY=/home/casparcg/.Xauthority export DBUS_SESSION_BUS_ADDRESS="unix:path=/run/user/1001/bus" echo "$(date): Exports set" xset s off xset s noblank xset -dpms xsetroot -cursor_name none & unclutter -idle 1 -root & echo "$(date): X configured" /usr/bin/gnome-keyring-daemon --start --components=secrets & sleep 1 nvidia-settings -a SyncToVBlank=1 & sleep 5 echo "$(date): Killing unwanted processes" pkill nm-applet 2>/dev/null pkill -f evolution-alarm 2>/dev/null pkill -f evolution-calendar 2>/dev/null pkill -f evolution-addressbook 2>/dev/null pkill -f evolution-source 2>/dev/null pkill -f system-config-printer 2>/dev/null echo "$(date): Starting scanner" bash -c 'cd /opt/casparcg && /usr/bin/casparcg-scanner >> /tmp/autostart-debug.log 2>&1' & sleep 2 echo "$(date): Starting CasparCG loop" while true; do cd /opt/casparcg/media/media/ pkill nm-applet 2>/dev/null pkill -f evolution-alarm 2>/dev/null rm -f /opt/casparcg/cef-cache/Singleton* rm -rf /tmp/.org.chromium.Chromium.* rm -rf /tmp/.com.google.* echo "$(date): Starting CasparCG" >> /tmp/casparcg-restart.log /usr/bin/casparcg-server-2.5 /opt/casparcg/media/casparcg.config.ftd >> /tmp/casparcg-restart.log 2>&1 EXIT_CODE=$? echo "$(date): CasparCG exited $EXIT_CODE" >> /tmp/casparcg-restart.log [ $EXIT_CODE -eq 0 ] && break while ss -tlnp ¦ grep -q 5250; do sleep 1; done sleep 5 done &

chmod +x /home/casparcg/.config/openbox/autostart # Exit back to serwer user exit

ℹ NOTE: The while-true loop with port-wait is critical. When a client sends RESTART, CasparCG exits cleanly. The loop waits for port 5250 to fully release, then relaunches automatically.

# 11. Syncthing for Media Sync

## 11.1 Install Syncthing

👤 Run as: root/serwer

sudo apt install -y syncthing # Stop any instance running as serwer user sudo systemctl stop syncthing@serwer 2>/dev/null ¦¦ true sudo systemctl disable syncthing@serwer 2>/dev/null ¦¦ true # Run as casparcg user so it has write access to /opt/casparcg/media/ sudo systemctl enable syncthing@casparcg sudo systemctl start syncthing@casparcg sudo systemctl status syncthing@casparcg

## 11.2 Configure Syncthing Port

👤 Run as: root/serwer

# Check which port the GUI is running on sudo systemctl status syncthing@casparcg ¦ grep GUI # Edit config to use standard port 8384 sudo -u casparcg nano /home/casparcg/.config/syncthing/config.xml # Find and change: # <address>127.0.0.1:XXXXX</address> # To: # <address>127.0.0.1:8384</address> sudo systemctl restart syncthing@casparcg

## 11.3 Access GUI from Remote Mac

Create an SSH tunnel from your Mac terminal:

ssh -L 8384:localhost:8384 serwer@<ubuntu-ip> # Then open in browser: # http://localhost:8384

## 11.4 Configure Media Folder in GUI

- Open http://localhost:8384 via SSH tunnel

- Remove the default 'Default Folder'

- Add folder: path = /opt/casparcg/media/

- Add your Mac as a remote device

- Accept the connection on the Mac side

- Set folder to Send & Receive on both sides

ℹ NOTE: Syncthing runs as casparcg so it has full write access to /opt/casparcg/media/. Never run it as serwer or root — permission denied errors will occur.

# 12. Network and Firewall

## 12.1 Fix Slow Boot (2 Minute Network Wait)

👤 Run as: root/serwer

sudo systemctl mask systemd-networkd-wait-online.service sudo systemctl mask NetworkManager-wait-online.service # Verify boot time after next reboot systemd-analyze blame ¦ head -20

## 12.2 Allow AMCP Port Through Firewall

👤 Run as: root/serwer

# Check status sudo ufw status # Allow AMCP from anywhere sudo ufw allow 5250/tcp # Or restrict to specific client IP (more secure) sudo ufw allow from 192.168.0.30 to any port 5250 proto tcp sudo ufw reload

# 13. Reboot and Verification

## 13.1 Full Reboot

👤 Run as: root/serwer

sudo reboot

Boot sequence should be: NVIDIA loads → LightDM starts → casparcg auto-login → Xorg :0 → Openbox → autostart runs → Scanner + CasparCG start.

## 13.2 Verify After Reboot (via SSH as serwer)

👤 Run as: root/serwer

# 1. Check single CasparCG process (CEF/zygote/utility processes are normal) ps aux ¦ grep casparcg-server ¦ grep -v grep ¦ grep -v zygote ¦ grep -v type # 2. Port 5250 listening on all interfaces ss -tlnp ¦ grep 5250 # Expected: LISTEN 0 4096 0.0.0.0:5250 # 3. Scanner running ps aux ¦ grep casparcg-scanner ¦ grep -v grep # 4. GPU active nvidia-smi # 5. DeckLink devices ls /dev/blackmagic/ # Expected: io0 io1 io2 io3 # 6. Check today's log tail -50 /opt/casparcg/log/caspar_$(date +%Y-%m-%d).log

## 13.3 Expected Log on Successful Start

Initialized OpenGL 4.6.0 NVIDIA 590.48.01 NVIDIA Corporation Decklink devices found: - DeckLink 8K Pro [1] (1637825600) - DeckLink 8K Pro [2] (1637825601) - DeckLink 8K Pro [3] (1637825602) - DeckLink 8K Pro [4] (1637825603) Initialized decklink module. Initialized newtek module. Loaded "/opt/casparcg/libndi.so.6" ndi_consumer Initialized. Screen consumer Enabled vsync. async_event_server[:5250] ...

## 13.4 Verification Checklist

| Check | Expected Result |
| --- | --- |
| Single CasparCG process | One entry (not counting CEF/zygote/utility) |
| Port 5250 open | ss -tlnp shows 0.0.0.0:5250 LISTEN |
| DeckLink detected | 4 devices: DeckLink 8K Pro [1-4] in log |
| NDI loaded | Log: Loaded /opt/casparcg/libndi.so.6 |
| vsync enabled | Log: Screen consumer Enabled vsync |
| Logs in correct path | /opt/casparcg/log/caspar_YYYY-MM-DD.log |
| Scanner running | ps aux shows casparcg-scanner process |
| Display stays on | xset q shows DPMS Disabled |
| RESTART works | Server comes back up within 5 seconds |
| Syncthing syncing | Media files appear in /opt/casparcg/media/ |

# 14. Operation Rules

⚠ WARNING: Breaking these rules causes X display errors, GPU failures, or crashes that require physical access to the server to fix.

## 14.1 DO

- Manage CasparCG via your AMCP client application

- Push config changes via DATA STORE casparcg.config from client (the .ftd extension is appended automatically)

- Send RESTART from client to apply new config — server will restart automatically via the while loop

- Use SSH only for system maintenance (logs, updates, troubleshooting)

- Monitor via: tail -f /opt/casparcg/log/caspar_$(date +%Y-%m-%d).log

- Add media by syncing via Syncthing to /opt/casparcg/media/

## 14.2 DO NOT

- Start CasparCG from SSH (no DISPLAY = instant crash)

- Run CasparCG from TTY

- Start CasparCG as root or serwer user

- Create a systemd service for CasparCG

- Install GNOME or switch to Wayland

- Change the working directory away from /opt/casparcg/ in the launch script

- Set scanner and server to the same port (scanner = 8000, server = 5250)

## 14.3 Config Push Workflow

- Client connects to CasparCG on port 5250

- Client sends: DATA STORE casparcg.config "<configuration>..."

- Server saves to: /opt/casparcg/media/casparcg.config.ftd

- Client sends: RESTART

- Server shuts down cleanly and exits

- Openbox while-loop waits for port 5250 to fully release

- CasparCG relaunches automatically with new config from /opt/casparcg/media/casparcg.config.ftd

- Client reconnects on port 5250 (allow 5-10 seconds)

# 15. Troubleshooting

## 15.1 Port 5250 Already in Use After Boot

# Check for rogue systemd service systemctl list-units ¦ grep -i casparcg ls /etc/systemd/system/ ¦ grep -i casparcg # If found: sudo systemctl disable casparcg-server sudo rm /etc/systemd/system/casparcg-server.service sudo systemctl daemon-reload sudo reboot

## 15.2 CasparCG Crashes with 'Failed to open X11 display'

# This means CasparCG was started outside of an X session # It CANNOT run from SSH or TTY # Verify LightDM is running and casparcg is logged in: systemctl status lightdm loginctl list-sessions # Verify autologin config: cat /etc/lightdm/lightdm.conf

## 15.3 Logs Not Appearing After RESTART

# Working directory is wrong — logs go to relative path log/ # Must be /opt/casparcg/log/ not /opt/casparcg-server-2.5/bin/log/ # Find where logs actually went: find / -name 'caspar_*.log' 2>/dev/null # Fix: ensure autostart uses 'cd /opt/casparcg/media/media/' before launching

## 15.4 NDI Not Working

ls /opt/casparcg/libndi.so.6 ldconfig -p ¦ grep libndi # If missing: sudo cp /usr/lib/x86_64-linux-gnu/libndi.so.6 /opt/casparcg/ sudo chown casparcg:casparcg /opt/casparcg/libndi.so.6

## 15.5 DeckLink Not Detected

lsmod ¦ grep blackmagic ls /dev/blackmagic/ # Reload module: sudo modprobe blackmagic_io # Verify driver version matches kernel: uname -r dpkg -l ¦ grep desktopvideo

## 15.6 Syncthing Permission Denied

# Syncthing must run as casparcg user ps aux ¦ grep syncthing ¦ grep -v grep # Must show: casparcg ... /usr/bin/syncthing # If running as wrong user: sudo systemctl stop syncthing@serwer sudo systemctl disable syncthing@serwer sudo systemctl enable syncthing@casparcg sudo systemctl start syncthing@casparcg # Fix permissions: sudo chown -R casparcg:casparcg /opt/casparcg/media/

## 15.7 Test AMCP Connection

# From Mac terminal: nc -v <ubuntu-ip> 5250 # Type a command: INFO # Should respond with server info XML

CasparCG Production Setup Guide — End of Document